FRC Team 250 Robot Code

Complete Technical Documentation

2026 Season - Rebuilt Codebase

# 1. WPILib Framework

WPILib is the core library for FRC robot programming, providing the foundation for all robot control systems.

## 1.1 Core Concepts

### Command-Based Programming

WPILib uses a command-based architecture that separates robot actions (Commands) from robot hardware (Subsystems). This provides:

- Clean separation of concerns

- Reusable command patterns

- Automatic scheduling and conflict resolution

- Built-in safety mechanisms

// Example Command structure
public class ExampleCommand extends Command {
  private final ExampleSubsystem subsystem;
  public ExampleCommand(ExampleSubsystem subsystem) {
    this.subsystem = subsystem;
    addRequirements(subsystem);
  }
  @Override
  public void initialize() {
    // Setup when command starts
  }
  @Override
  public void execute() {
    // Runs repeatedly while command is scheduled
  }
  @Override
  public void end(boolean interrupted) {
    // Cleanup when command ends
  }
  @Override
  public boolean isFinished() {
    // Return true when command should end
    return false;
  }
}

### Subsystems

Subsystems represent physical robot mechanisms and manage their state.

public class ExampleSubsystem extends SubsystemBase {
  private final CANSparkMax motor;
  public ExampleSubsystem() {
    motor = new CANSparkMax(1, MotorType.kBrushless);
  }
  @Override
  public void periodic() {
    // Runs every 20ms robot loop
  }
  public void setSpeed(double speed) {
    motor.set(speed);
  }
}

## 1.2 Key WPILib Classes

| Class | Purpose |
| --- | --- |
| TimedRobot | Main robot class, runs periodic methods |
| Command | Base class for robot actions |
| SubsystemBase | Base class for robot mechanisms |
| CommandScheduler | Manages command execution |
| Trigger | Binds buttons to commands |
| NetworkTables | Inter-process communication |
| SmartDashboard | Dashboard telemetry |
| Field2d | Visualize robot position |

## 1.3 Control Systems

### PID Controllers

Proportional-Integral-Derivative controllers for closed-loop control.

// PID controller setup
PIDController pidController = new PIDController(kP, kI, kD);
pidController.setTolerance(0.05); // 5% tolerance
// In execute() method
double output = pidController.calculate(
  measurement,  // Current position
  setpoint      // Desired position
);
motor.set(output);

| Term | Effect | When to Use |
| --- | --- | --- |
| P (Proportional) | Responds to current error | Primary correction force |
| I (Integral) | Responds to accumulated error | Eliminate steady-state error |
| D (Derivative) | Responds to rate of change | Reduce oscillation/overshoot |

### Feedforward Control

Feedforward compensates for known system dynamics before feedback correction.

// Simple feedforward
SimpleMotorFeedforward feedforward = 
  new SimpleMotorFeedforward(kS, kV, kA);
// Calculate feedforward voltage
double ff = feedforward.calculate(
  currentVelocity,
  nextVelocity,
  dt
);
// Combine with PID
motor.setVoltage(pidOutput + ff);

# 2. REV Robotics Library

REV provides motor controllers (SPARK MAX, SPARK Flex) and sensors for FRC robots.

## 2.1 SPARK Motor Controllers

### CANSparkMax and CANSparkFlex

// Initialize SPARK motor controller
CANSparkMax motor = new CANSparkMax(
  deviceID,           // CAN ID (1-62)
  MotorType.kBrushless  // NEO/NEO 550 motors
);
// Configuration
motor.restoreFactoryDefaults();
motor.setIdleMode(IdleMode.kBrake);
motor.setSmartCurrentLimit(40); // Amps
motor.enableVoltageCompensation(12.0);
motor.setInverted(false);
// Save configuration to flash
motor.burnFlash();

| Method | Purpose |
| --- | --- |
| set(double speed) | Set motor output (-1.0 to 1.0) |
| setVoltage(double volts) | Set output voltage |
| stopMotor() | Stop motor immediately |
| setIdleMode(IdleMode) | Brake or Coast when neutral |
| setSmartCurrentLimit(int) | Current limiting (amps) |
| setInverted(boolean) | Reverse motor direction |
| follow(CANSparkBase) | Follower mode |
| burnFlash() | Save config to non-volatile memory |

### Encoders

// Get built-in encoder
RelativeEncoder encoder = motor.getEncoder();
// Encoder methods
double position = encoder.getPosition();      // Rotations
double velocity = encoder.getVelocity();      // RPM
encoder.setPosition(0);                       // Zero position
encoder.setPositionConversionFactor(factor);  // Scale output
encoder.setVelocityConversionFactor(factor);  // Scale velocity
// Example: convert to linear distance
// If wheel diameter = 4 inches, gear ratio = 6.75:1
double wheelCircumference = 4 * Math.PI;
double gearRatio = 6.75;
encoder.setPositionConversionFactor(
  wheelCircumference / gearRatio  // inches per motor rotation
);

### PID Control (On-Controller)

// Get PID controller
SparkPIDController pidController = motor.getPIDController();
// Configure PID constants (Slot 0)
pidController.setP(kP, 0);
pidController.setI(kI, 0);
pidController.setD(kD, 0);
pidController.setFF(kFF, 0);
pidController.setOutputRange(-1, 1, 0);
// Set reference
pidController.setReference(
  setpoint,                    // Target value
  ControlType.kVelocity,       // Control mode
  0,                           // PID slot
  feedforward,                 // Arbitrary feedforward
  ArbFFUnits.kVoltage          // FF units
);

| ControlType | Use Case |
| --- | --- |
| kDutyCycle | Open-loop percent output |
| kVelocity | Velocity control (RPM) |
| kPosition | Position control (rotations) |
| kVoltage | Voltage control |
| kSmartMotion | Motion profiling |
| kSmartVelocity | Velocity with acceleration limiting |

## 2.2 MAXSwerve Integration

MAXSwerve modules use SPARK controllers with integrated absolute encoders.

// MAXSwerve module configuration
public class MAXSwerveModule {
  private final CANSparkMax drivingMotor;
  private final CANSparkMax turningMotor;
  private final RelativeEncoder drivingEncoder;
  private final AbsoluteEncoder turningEncoder;
  private final SparkPIDController drivingPID;
  private final SparkPIDController turningPID;
  public MAXSwerveModule(int drivingID, int turningID) {
    drivingMotor = new CANSparkMax(drivingID, MotorType.kBrushless);
    turningMotor = new CANSparkMax(turningID, MotorType.kBrushless);
    // Get absolute encoder for turning
    turningEncoder = turningMotor.getAbsoluteEncoder(
      SparkAbsoluteEncoder.Type.kDutyCycle
    );
    turningEncoder.setPositionConversionFactor(2 * Math.PI); // radians
    // Get relative encoders
    drivingEncoder = drivingMotor.getEncoder();
    // Configure PID
    drivingPID = drivingMotor.getPIDController();
    turningPID = turningMotor.getPIDController();
    turningPID.setPositionPIDWrappingEnabled(true); // Continuous
    turningPID.setPositionPIDWrappingMinInput(-Math.PI);
    turningPID.setPositionPIDWrappingMaxInput(Math.PI);
  }
}

# 3. AdvantageKit Framework

AdvantageKit provides deterministic logging and replay capabilities for robot code.

## 3.1 Core Architecture

### IO Pattern

AdvantageKit uses the IO pattern to separate hardware interface from logic:

- IO Interface: Defines inputs/outputs

- IO Implementation: Hardware-specific code

- Subsystem: Contains logic, uses IO interface

// IO Interface
public interface DriveIO {
  @AutoLog
  public static class DriveIOInputs {
    public double leftPositionMeters = 0.0;
    public double rightPositionMeters = 0.0;
    public double leftVelocityMetersPerSec = 0.0;
    public double rightVelocityMetersPerSec = 0.0;
    public double[] currentAmps = new double[] {};
  }
  public default void updateInputs(DriveIOInputs inputs) {}
  public default void setVoltage(double leftVolts, double rightVolts) {}
}
// Hardware Implementation
public class DriveIOSparkMax implements DriveIO {
  private final CANSparkMax leftMotor;
  private final CANSparkMax rightMotor;
  private final RelativeEncoder leftEncoder;
  private final RelativeEncoder rightEncoder;
  @Override
  public void updateInputs(DriveIOInputs inputs) {
    inputs.leftPositionMeters = leftEncoder.getPosition();
    inputs.rightPositionMeters = rightEncoder.getPosition();
    inputs.leftVelocityMetersPerSec = leftEncoder.getVelocity();
    inputs.rightVelocityMetersPerSec = rightEncoder.getVelocity();
  }
  @Override
  public void setVoltage(double leftVolts, double rightVolts) {
    leftMotor.setVoltage(leftVolts);
    rightMotor.setVoltage(rightVolts);
  }
}
// Subsystem
public class Drive extends SubsystemBase {
  private final DriveIO io;
  private final DriveIOInputsAutoLogged inputs = new DriveIOInputsAutoLogged();
  public Drive(DriveIO io) {
    this.io = io;
  }
  @Override
  public void periodic() {
    io.updateInputs(inputs);
    Logger.processInputs("Drive", inputs);
  }
}

### Simulation IO

public class DriveIOSim implements DriveIO {
  private final DCMotorSim leftSim;
  private final DCMotorSim rightSim;
  private double leftAppliedVolts = 0.0;
  private double rightAppliedVolts = 0.0;
  public DriveIOSim() {
    leftSim = new DCMotorSim(
      DCMotor.getNEO(2),     // 2x NEO motors
      6.75,                  // Gear ratio
      0.025                  // MOI kg*m^2
    );
    rightSim = new DCMotorSim(DCMotor.getNEO(2), 6.75, 0.025);
  }
  @Override
  public void updateInputs(DriveIOInputs inputs) {
    leftSim.update(0.02);   // 20ms period
    rightSim.update(0.02);
    inputs.leftPositionMeters = leftSim.getAngularPositionRotations();
    inputs.rightPositionMeters = rightSim.getAngularPositionRotations();
    inputs.leftVelocityMetersPerSec = leftSim.getAngularVelocityRPM() / 60.0;
    inputs.rightVelocityMetersPerSec = rightSim.getAngularVelocityRPM() / 60.0;
  }
  @Override
  public void setVoltage(double leftVolts, double rightVolts) {
    leftAppliedVolts = MathUtil.clamp(leftVolts, -12.0, 12.0);
    rightAppliedVolts = MathUtil.clamp(rightVolts, -12.0, 12.0);
    leftSim.setInputVoltage(leftAppliedVolts);
    rightSim.setInputVoltage(rightAppliedVolts);
  }
}

## 3.2 Logging

AdvantageKit automatically logs all inputs marked with @AutoLog.

// In Robot.java
@Override
public void robotInit() {
  Logger.recordMetadata("ProjectName", "Team250-2026");
  Logger.recordMetadata("BuildDate", BuildConstants.BUILD_DATE);
  Logger.recordMetadata("GitSHA", BuildConstants.GIT_SHA);
  if (isReal()) {
    Logger.addDataReceiver(new WPILOGWriter("/U/logs"));
    Logger.addDataReceiver(new NT4Publisher());
  } else {
    Logger.addDataReceiver(new WPILOGWriter(""));
  }
  Logger.start();
}
// Manual logging
Logger.recordOutput("Drive/LeftVelocitySetpoint", velocitySetpoint);
Logger.recordOutput("Drive/LeftVelocityMeasured", inputs.leftVelocityMetersPerSec);

## 3.3 Log Replay

Replay logged data to test code changes without a physical robot:

- Run robot code normally, logs are saved

- Modify control logic in subsystems

- Run simulation with log file as input

- Compare outputs in AdvantageScope

# 4. PathPlanner Integration

PathPlanner provides autonomous path planning and following for swerve drive robots.

## 4.1 Auto Builder Configuration

// In Drive subsystem constructor
AutoBuilder.configureHolonomic(
  this::getPose,              // Robot pose supplier
  this::resetPose,            // Pose reset method
  this::getChassisSpeeds,     // Current speeds supplier
  this::runVelocity,          // Drive method
  new HolonomicPathFollowerConfig(
    new PIDConstants(5.0, 0.0, 0.0),      // Translation PID
    new PIDConstants(5.0, 0.0, 0.0),      // Rotation PID
    MAX_LINEAR_SPEED,
    DRIVE_BASE_RADIUS,
    new ReplanningConfig()
  ),
  () -> {
    var alliance = DriverStation.getAlliance();
    return alliance.isPresent() && alliance.get() == Alliance.Red;
  },
  this
);

## 4.2 Creating Auto Commands

// Load path from deploy directory
PathPlannerPath path = PathPlannerPath.fromPathFile("MyPath");
// Follow path command
Command followPath = AutoBuilder.followPath(path);
// Use in auto routine
return Commands.sequence(
  new PrepareShooterCommand(shooter),
  new ShootCommand(shooter),
  followPath,
  new IntakeCommand(intake)
);

## 4.3 Named Commands

// Register named commands (in RobotContainer)
NamedCommands.registerCommand("Shoot", new ShootCommand(shooter));
NamedCommands.registerCommand("Intake", new IntakeCommand(intake));
// Use in PathPlanner GUI
// Paths can trigger these commands at waypoints

# 5. Team 250 Custom Code

Team 250's 2026 robot codebase built on AdvantageKit Spark Swerve Template.

## 5.1 Project Structure

| Package | Purpose |
| --- | --- |
| frc.robot | Main robot class and constants |
| frc.robot.subsystems | Hardware subsystems (Drive, Shooter, etc) |
| frc.robot.commands | Robot actions and auto routines |
| frc.robot.vision | Vision processing integration |
| frc.robot.util | Helper classes and utilities |

## 5.2 Drive Subsystem

### Swerve Drive Implementation

public class Drive extends SubsystemBase {
  private final GyroIO gyroIO;
  private final GyroIOInputsAutoLogged gyroInputs = new GyroIOInputsAutoLogged();
  private final Module[] modules = new Module[4];
  private SwerveDriveKinematics kinematics;
  private SwerveDrivePoseEstimator poseEstimator;
  private Field2d field = new Field2d();
  public Drive(GyroIO gyroIO, ModuleIO fl, ModuleIO fr, 
               ModuleIO bl, ModuleIO br) {
    this.gyroIO = gyroIO;
    modules[0] = new Module(fl, 0);
    modules[1] = new Module(fr, 1);
    modules[2] = new Module(bl, 2);
    modules[3] = new Module(br, 3);
    // Configure kinematics
    kinematics = new SwerveDriveKinematics(
      new Translation2d(TRACK_WIDTH / 2, WHEEL_BASE / 2),   // FL
      new Translation2d(TRACK_WIDTH / 2, -WHEEL_BASE / 2),  // FR
      new Translation2d(-TRACK_WIDTH / 2, WHEEL_BASE / 2),  // BL
      new Translation2d(-TRACK_WIDTH / 2, -WHEEL_BASE / 2)  // BR
    );
  }
  public void runVelocity(ChassisSpeeds speeds) {
    // Convert to module states
    SwerveModuleState[] states = kinematics.toSwerveModuleStates(speeds);
    SwerveDriveKinematics.desaturateWheelSpeeds(states, MAX_LINEAR_SPEED);
    // Set module states
    for (int i = 0; i < 4; i++) {
      modules[i].runSetpoint(states[i]);
    }
  }
}

### Module Class

public class Module {
  private final ModuleIO io;
  private final ModuleIOInputsAutoLogged inputs = 
    new ModuleIOInputsAutoLogged();
  private final int index;
  private final SimpleMotorFeedforward driveFeedforward;
  private final PIDController driveFeedback;
  private final PIDController turnFeedback;
  public Module(ModuleIO io, int index) {
    this.io = io;
    this.index = index;
    driveFeedforward = new SimpleMotorFeedforward(0.1, 0.13);
    driveFeedback = new PIDController(0.05, 0.0, 0.0);
    turnFeedback = new PIDController(7.0, 0.0, 0.0);
    turnFeedback.enableContinuousInput(-Math.PI, Math.PI);
  }
  public void runSetpoint(SwerveModuleState state) {
    // Optimize to avoid >90° rotation
    state = SwerveModuleState.optimize(state, 
      new Rotation2d(inputs.turnPosition));
    // Drive PID + FF
    double driveFF = driveFeedforward.calculate(state.speedMetersPerSecond);
    double driveFB = driveFeedback.calculate(
      inputs.driveVelocityMetersPerSec, 
      state.speedMetersPerSecond
    );
    // Turn PID
    double turnFB = turnFeedback.calculate(
      inputs.turnPosition, 
      state.angle.getRadians()
    );
    io.setDriveVoltage(driveFF + driveFB);
    io.setTurnVoltage(turnFB);
  }
}

## 5.3 Vision Integration

### FuelDetection Subsystem

public class FuelDetection extends SubsystemBase {
  private NetworkTable visionTable;
  private NetworkTableEntry ballXEntry;
  private NetworkTableEntry ballYEntry;
  private NetworkTableEntry ballDistanceEntry;
  public FuelDetection() {
    visionTable = NetworkTableInstance.getDefault()
      .getTable("Vision26");
    ballXEntry = visionTable.getEntry("ball_x");
    ballYEntry = visionTable.getEntry("ball_y");
    ballDistanceEntry = visionTable.getEntry("ball_distance");
  }
  public Optional<BallData> getNearestBall() {
    double x = ballXEntry.getDouble(-1);
    double y = ballYEntry.getDouble(-1);
    double distance = ballDistanceEntry.getDouble(-1);
    if (x >= 0 && y >= 0 && distance > 0) {
      return Optional.of(new BallData(x, y, distance));
    }
    return Optional.empty();
  }
  public static class BallData {
    public final double x;      // pixels
    public final double y;      // pixels
    public final double distance; // meters
    public BallData(double x, double y, double distance) {
      this.x = x;
      this.y = y;
      this.distance = distance;
    }
  }
}

### Python Vision Pipeline

Raspberry Pi 5 runs YOLOv8 detection with NCNN inference:

# vision_pipeline.py
import cv2
import numpy as np
from ncnn_yolo import YOLOv8
from networktables import NetworkTablesInstance
def main():
    # Initialize NetworkTables
    nt = NetworkTablesInstance.getDefault()
    nt.startClient4("raspberrypi")
    nt.setServerTeam(250)
    vision_table = nt.getTable("Vision26")
    # Load model
    detector = YOLOv8("best_ncnn_model")
    # Camera setup
    cap = cv2.VideoCapture(0)
    while True:
        ret, frame = cap.read()
        if not ret:
            continue
        # Undistort fisheye image
        frame = cv2.fisheye.undistortImage(frame, K, D)
        # Run detection
        results = detector.detect(frame)
        # Find nearest ball
        nearest = find_nearest_ball(results)
        if nearest:
            vision_table.putNumber("ball_x", nearest.x)
            vision_table.putNumber("ball_y", nearest.y)
            vision_table.putNumber("ball_distance", 
              estimate_distance(nearest))
        else:
            vision_table.putNumber("ball_x", -1)
            vision_table.putNumber("ball_y", -1)
            vision_table.putNumber("ball_distance", -1)

## 5.4 Shooter Subsystem

public class Shooter extends SubsystemBase {
  private final ShooterIO io;
  private final ShooterIOInputsAutoLogged inputs = 
    new ShooterIOInputsAutoLogged();
  private double targetRPM = 0.0;
  public Shooter(ShooterIO io) {
    this.io = io;
  }
  @Override
  public void periodic() {
    io.updateInputs(inputs);
    Logger.processInputs("Shooter", inputs);
    // Log setpoint
    Logger.recordOutput("Shooter/TargetRPM", targetRPM);
  }
  public void setVelocity(double rpm) {
    targetRPM = rpm;
    io.setVelocity(rpm);
  }
  public boolean atSetpoint() {
    return Math.abs(inputs.velocityRPM - targetRPM) < 50;
  }
  public void stop() {
    targetRPM = 0.0;
    io.stop();
  }
}

## 5.5 State Machine Commands

### FuelTrackingCommand

public class FuelTrackingCommand extends Command {
  private final Drive drive;
  private final FuelDetection fuelDetection;
  private enum State {
    SEARCHING,
    APPROACHING,
    ACQUIRING,
    COMPLETE
  }
  private State currentState = State.SEARCHING;
  public FuelTrackingCommand(Drive drive, FuelDetection fuelDetection) {
    this.drive = drive;
    this.fuelDetection = fuelDetection;
    addRequirements(drive, fuelDetection);
  }
  @Override
  public void execute() {
    Optional<BallData> ball = fuelDetection.getNearestBall();
    switch (currentState) {
      case SEARCHING:
        if (ball.isPresent()) {
          currentState = State.APPROACHING;
        } else {
          // Spin to search
          drive.runVelocity(new ChassisSpeeds(0, 0, 0.5));
        }
        break;
      case APPROACHING:
        if (!ball.isPresent()) {
          currentState = State.SEARCHING;
          break;
        }
        BallData b = ball.get();
        if (b.distance < 0.5) { // 0.5m threshold
          currentState = State.ACQUIRING;
        } else {
          // Drive toward ball
          drive.runVelocity(calculateApproachSpeeds(b));
        }
        break;
      case ACQUIRING:
        // Run intake, wait for sensor
        if (intakeSensor.get()) {
          currentState = State.COMPLETE;
        }
        break;
    }
  }
  @Override
  public boolean isFinished() {
    return currentState == State.COMPLETE;
  }
}

# 6. Best Practices

## 6.1 Code Organization

- Use AdvantageKit IO pattern for all hardware

- Keep subsystems focused on single mechanisms

- Put complex logic in commands, not subsystems

- Always create simulation IO implementations

## 6.2 Safety

- Always set current limits on motors

- Use MathUtil.clamp() for all output values

- Enable voltage compensation (12V nominal)

- Test in simulation before deploying to robot

## 6.3 Tuning

| Component | Tuning Order |
| --- | --- |
| Motor direction | 1. Verify positive = desired direction |
| Encoder scaling | 2. Verify position/velocity units |
| Feedforward | 3. Get basic motion without oscillation |
| P gain | 4. Increase until oscillation, reduce by 50% |
| D gain | 5. Add to reduce overshoot |
| I gain | 6. Only if steady-state error remains |

## 6.4 Logging

- Log all sensor inputs via AdvantageKit

- Log setpoints alongside measurements

- Log command states and transitions

- Review logs after every test run

# 7. Common Issues & Solutions

## 7.1 Motor Issues

| Problem | Solution |
| --- | --- |
| Motor not responding | Check CAN ID, verify power, check firmware |
| Motor runs backward | Use setInverted(true) on motor |
| Encoder not updating | Check encoder type, verify motor rotation |
| Current limiting | Increase setSmartCurrentLimit() or check load |
| Inconsistent behavior | Enable voltage compensation |

## 7.2 Swerve Issues

| Problem | Solution |
| --- | --- |
| Modules fighting | Check that all wheels point forward at 0° |
| Rotation overshoot | Tune rotation PID, reduce D gain |
| Drift during auto | Check gyro calibration, verify field-relative |
| Jerky movement | Enable desaturation, check feedforward |
| Wrong direction | Verify Translation2d signs in kinematics |

## 7.3 Vision Issues

| Problem | Solution |
| --- | --- |
| No NetworkTables data | Check team number, verify Pi network |
| Incorrect distance | Recalibrate camera, verify focal length |
| Detection lag | Reduce resolution, optimize NCNN model |
| False detections | Increase confidence threshold, retrain model |
| Pi not booting | Check SD card, verify power supply (5V 3A+) |

## 7.4 PathPlanner Issues

| Problem | Solution |
| --- | --- |
| Path not following | Check AutoBuilder configuration |
| Rotation overshoot | Reduce rotation PID gains |
| Path not found | Verify path file in deploy/pathplanner/paths/ |
| Wrong alliance | Check alliance color supplier in AutoBuilder |
| Stops mid-path | Verify max velocity/acceleration constraints |

# 8. Quick Reference

## 8.1 Units Conversion

| From | To | Formula |
| --- | --- | --- |
| Rotations | Radians | rotations × 2π |
| RPM | Rad/s | rpm × 2π / 60 |
| RPM | m/s (linear) | (rpm / 60) × 2π × radius |
| Meters | Inches | meters × 39.37 |
| Degrees | Radians | degrees × π / 180 |

## 8.2 Common Constants

| Constant | Value | Use |
| --- | --- | --- |
| Robot loop period | 0.02 s (20ms) | Update rate for all periodic methods |
| CAN timeout | 0 ms | Use 0 for non-blocking CAN calls |
| Max voltage | 12.0 V | Nominal battery voltage |
| NEO free speed | 5676 RPM | Theoretical max (use ~95% in practice) |
| NEO stall current | 105 A | Set current limit below this |
| Wheel diameter (typical) | 4 inches | Depends on wheel choice |

## 8.3 Useful Links

- WPILib Documentation: https://docs.wpilib.org

- REV Robotics Docs: https://docs.revrobotics.com

- AdvantageKit: https://github.com/Mechanical-Advantage/AdvantageKit

- PathPlanner: https://pathplanner.dev

- PhotonVision: https://docs.photonvision.org

- Chief Delphi: https://chiefdelphi.com

# Appendix A: Competition Checklist

## Pre-Match

- Verify battery voltage >12.5V

- Check all CAN devices show in Phoenix Tuner

- Calibrate gyro (robot stationary for 10s)

- Test all subsystems in teleop

- Verify auto path loaded correctly

- Disable WiFi on vision coprocessor

## Post-Match

- Download logs from roboRIO

- Check for CAN errors or brownouts

- Review AdvantageScope for anomalies

- Document any issues for next match

# Appendix B: Git Workflow

Team 250 uses feature branches and pull requests for code review.

# Create feature branch
git checkout -b feature/shooter-improvements
# Make changes and commit
git add .
git commit -m "Add feedforward to shooter control"
# Push to GitHub
git push origin feature/shooter-improvements
# Create pull request on GitHub
# Request review from team members
# Merge after approval
# Update local main
git checkout main
git pull origin main

## Commit Message Guidelines

- Use present tense: 'Add feature' not 'Added feature'

- Be descriptive: Include what and why

- Reference issues: 'Fix #123: Resolve rotation overshoot'

End of Documentation

Team 250 • 2026 Season